Ok well I assume you want to program Bank Switching along with self-level and Rescue correct ?
I that's correct then here my setup

I have Bank switching on Switch C (its your choice of 3 position switch)
Bank 0 is Normal Self Level
Bank 1 is Normal Rescue
Bank 2 is Acro Self Level

Let’s start with the Radio
Power on Radio Holding down the roller until powered up
Then go to Channel Assign and assign Aux 2 to your switch in my case C
Then go Model Main screen 
Click roller and scroll down to Gyro and assign it to the same switch.
And set position 0 to -100
Position 1 to 0
Position 3 to +100
Then we need to set up 3 mixes
Mix 1 and Mix 2 will be EXACTLY the same 
GER>GYR
Rate -100 % 0%
Offset 0
Switch I White Box 0 Black Box 1
Mix 3
Aux2>Aux2
Rate -100% -100%
Offset 0
Switch I White Box 0 Black Box 1

Now in the software
General Tab
Channel Assign
Gyro Gain Channel 5
Bank Channel 7
The I Diagnostic Tab
Verify Bank switching follows 3 position Switch
reverse in radio if necessary
Channel 5 verify 0 and +/-100 use travel adjust for the 100 and sub trim for the 0 readings. This is the ONLY time you use Sub Trim.
Then in radio in Gyro set position 0 and 2 to -60 and position 1 to +60
In Stabi Tab Select Bank 0 lower right corner 
Select Normal Stabilization
Bank 1 Select Normal Rescue 
Bank 2 Acro Stabilization.
How it works is when Switch E in 0 and 2 you have Neg Gyro gain and Stabilization Mode
In position1 you have Pos Gyro Gain Normal Flight NO stabilization.
When in Position 1 and press switch I Gain changes to Neg and have rescue.
When in Position 0 and 2 and press switch I you change to Bank 1 and Gain Neg and have rescue.
When you get it all done we can then verify you have it all programmed correctly.
This can be a lot to digest and confusing, It was for me at first also.
So go through it and let me know if you have any questions.